# Wiper (two-bit) DE1-SoC I/O mappings

All signals are active high. GPIO numbers refer to the LabsLand virtual GPIO bus; in VHDL, for example, GPIO 28 is V_GPIO(28).

| Runtime signal | Meaning in the current simulation | Type | GPIO |
| --- | --- | --- | --- |
| rainSensor | Rain is active | FPGA input | GPIO 28 (V_GPIO(28)) |
| rightSensor | Wiper is at the left-hand limit | FPGA input | GPIO 29 (V_GPIO(29)) |
| leftSensor | Wiper is at the right-hand limit | FPGA input | GPIO 30 (V_GPIO(30)) |
| mButton | The interface’s M button is being pressed | FPGA input | GPIO 23 (V_GPIO(23)) |
| pButton | The interface’s P button is being pressed | FPGA input | GPIO 24 (V_GPIO(24)) |
| move1 | First movement-control signal | FPGA output | GPIO 26 (V_GPIO(26)) |
| move2 | Second movement-control signal | FPGA output | GPIO 27 (V_GPIO(27)) |

The command pair is written in (move1, move2) order:

| move1 / GPIO 26 | move2 / GPIO 27 | Result |
| --- | --- | --- |
| 0 | 0 | Stop |
| 0 | 1 | Move toward the right-hand limit |
| 1 | 0 | Move toward the left-hand limit |
| 1 | 1 | Error state |

The current two-bit simulation publishes the endpoint channels with their runtime names reversed: rightSensor reports the physical left-hand limit and leftSensor reports the physical right-hand limit. This is different from the one-bit Wiper. Use the endpoint meanings in the table above when implementing a controller. Between the endpoints, both endpoint sensors can be low.