# Wiper (two-bit) STM32 Nucleo WB55RG I/O mappings

All signals are active high. The direction shown is relative to the STM32.

| Runtime signal | Meaning in the current simulation | Type | GPIO | STM32 HAL name |
| --- | --- | --- | --- | --- |
| rainSensor | Rain is active | STM32 input | PC_13 | GPIOC, GPIO_PIN_13 |
| rightSensor | Wiper is at the left-hand limit | STM32 input | PA_6 | GPIOA, GPIO_PIN_6 |
| leftSensor | Wiper is at the right-hand limit | STM32 input | PB_9 | GPIOB, GPIO_PIN_9 |
| mButton | The interface’s M button is being pressed | STM32 input | PB_8 | GPIOB, GPIO_PIN_8 |
| pButton | The interface’s P button is being pressed | STM32 input | PC_12 | GPIOC, GPIO_PIN_12 |
| move1 | First movement-control signal | STM32 output | PC_4 | GPIOC, GPIO_PIN_4 |
| move2 | Second movement-control signal | STM32 output | PD_0 | GPIOD, GPIO_PIN_0 |

The command pair is written in (move1, move2) order:

| move1 / PC_4 | move2 / PD_0 | Result |
| --- | --- | --- |
| 0 | 0 | Stop |
| 0 | 1 | Move toward the right-hand limit |
| 1 | 0 | Move toward the left-hand limit |
| 1 | 1 | Error state |

The current two-bit simulation publishes the endpoint channels with their runtime names reversed: rightSensor reports the physical left-hand limit and leftSensor reports the physical right-hand limit. This is different from the one-bit Wiper. Use the endpoint meanings in the table above when implementing a controller. Between the endpoints, both endpoint sensors can be low.