# Wiper (one-bit) STM32 Nucleo WB55RG I/O mappings

All signals are active high. The direction shown is relative to the STM32.

| Runtime signal | Meaning | Type | GPIO | STM32 HAL name |
| --- | --- | --- | --- | --- |
| rainSensor | Rain is active | STM32 input | PC_13 | GPIOC, GPIO_PIN_13 |
| rightSensor | Wiper is at the right-hand limit (minimum angle) | STM32 input | PA_6 | GPIOA, GPIO_PIN_6 |
| leftSensor | Wiper is at the left-hand limit (maximum angle) | STM32 input | PB_9 | GPIOB, GPIO_PIN_9 |
| mButton | The interface’s M button is being pressed | STM32 input | PB_8 | GPIOB, GPIO_PIN_8 |
| pButton | The interface’s P button is being pressed | STM32 input | PC_12 | GPIOC, GPIO_PIN_12 |
| move | Enable wiper movement | STM32 output | PC_4 | GPIOC, GPIO_PIN_4 |

When move is high, the simulation moves the wiper and reverses its direction automatically at each endpoint. When move is low, it stops at its current position. Between the endpoints, both endpoint sensors can be low. The simulation’s error control deliberately makes both endpoint sensors high so that controller error handling can be tested.