REDTAIL LABORATORY EXERCISE

Rain-Responsive Wiper Controller

STM32 NUCLEO-WB55RG with Mbed OS

LEVEL  Basic     |     TIME  45-75 minutes     |     PLATFORM  STM32 NUCLEO-WB55RG

Implement and test a one-bit windshield-wiper controller on the STM32 NUCLEO-WB55RG. Your Mbed application decides whether the blade should move; the simulation owns its angle and automatic endpoint reversal.

The Wiper 3D simulation and original remote-laboratory exercise were created by Giovanna Lani and WebLab-Deusto at the University of Deusto in Spain. This lesson adapts their rain-driven challenge to the STM32/Mbed workflow and the current REDTAIL mapping. The completed application is available separately to verified instructors.

Figure 1. Wiper 3D simulation by Giovanna Lani and WebLab-Deusto, University of Deusto (Spain).

Figure 2. One-bit Wiper control loop.

# Learning objectives

After completing the activity, you should be able to:

- configure Mbed digital inputs and outputs for a remote simulation;

- implement a deterministic Boolean control policy;

- use one input snapshot per polling-loop iteration;

- distinguish controller logic from simulation-owned motion;

- stop conservatively on contradictory endpoint feedback;

- test rain, pause, resume, automatic reversal, and fault behavior.

# Prerequisites

You should know basic C++, Mbed DigitalIn and DigitalOut, and polling loops. You need an Mbed OS 6 or Mbed OS CE environment for NUCLEO_WB55RG and access to the Wiper simulation.

Open the REDTAIL Wiper simulation and the current STM32 mapping before starting.

# System behavior

| Input | Meaning in the current simulation |
| --- | --- |
| rainSensor | Rain is active and automatic wiping is requested. |
| rightSensor | The blade is within the right endpoint range. |
| leftSensor | The blade is within the left endpoint range. |
| mButton | Optional M button; not required by the core assignment. |
| pButton | Optional P button; not required by the core assignment. |

| Output | Meaning |
| --- | --- |
| move | High allows movement; low pauses the blade. |

The core application must move during rain, stop while dry, stop when both endpoint inputs are high, leave normal direction/reversal to the simulation, and ignore M/P.

# STM32 signal mapping

Directions are relative to the Mbed application.

| Runtime signal | Mbed pin | Direction |
| --- | --- | --- |
| Rain sensor | PC_13 | Input |
| Right endpoint | PA_6 | Input |
| Left endpoint | PB_9 | Input |
| M button | PB_8 | Input |
| P button | PC_12 | Input |
| Move | PC_4 | Output |

Optional LEDs on PB_13, PB_14, and PB_15 may mirror inputs or the move command. The current REDTAIL mapping is authoritative over older notes.

# Assignment

## 1. Derive a pure policy

Write a truth table for rain and both endpoint sensors. Separate the decision from GPIO access so it can be tested without hardware. Your policy must return one Boolean move request.

## 2. Create the Mbed application

Create main.cpp:

#include "mbed.h"
 
bool should_move(bool rain, bool right_endpoint, bool left_endpoint) {
    // Return the required one-bit policy.
}
 
int main() {
    // Configure five DigitalIn objects and one DigitalOut.
    // Initialize move low before entering the loop.
 
    while (true) {
        // Read one stable snapshot of all inputs.
        // Evaluate should_move and update the output.
        ThisThread::sleep_for(10ms);
    }
}

Do not repeatedly read a changing input while computing one output value. Initialize move low so startup cannot produce unintended motion.

## 3. Test the policy before programming

Exercise the normal and dual-endpoint rows independently of the GPIO loop. Confirm that changing M or P cannot alter the core result.

## 4. Compile and program

- Select NUCLEO_WB55RG, the Mbed environment, and the Wiper simulation.

- Compile and resolve every error or pin-name warning.

- Program the board and keep the simulation visible.

- Use diagnostic LEDs if needed to separate an input-mapping problem from a policy problem.

# Required test sequence

| Test | Action | Expected behavior |
| --- | --- | --- |
| Dry start | Start with rain disabled. | Move remains low. |
| Rain start | Enable rain. | The blade starts moving. |
| Automatic reversal | Keep rain enabled through both endpoints. | The simulation reverses the blade automatically. |
| Pause and resume | Disable rain between endpoints, then re-enable it. | The blade pauses and resumes. |
| Button isolation | Press M and P independently while dry. | The core application remains stopped. |
| Sensor fault | Trigger both endpoint sensors together. | Move becomes low until the fault clears. |
| Recovery | Clear the fault while rain remains enabled. | Movement resumes. |

# Deliverables

Submit the truth table, main.cpp, policy-test evidence, live test evidence, and a short explanation of the controller/simulation ownership boundary.

# Optional extensions

- Assign M or P a clearly documented manual-wipe behavior.

- Latch a fault indicator until an explicit acknowledgement.

- Report state transitions over serial without slowing the polling loop.

- Replace the delay with a periodic event while preserving one input snapshot.

# Completion checklist

- The target and all six pins match the current REDTAIL mapping.

- Move initializes low and every loop uses one input snapshot.

- Rain starts wiping and dry conditions stop it.

- The simulation owns automatic reversal.

- Dual endpoints stop safely and M/P remain optional.

- All source and evidence are ready to submit.