REDTAIL LABORATORY EXERCISE

Automatic Door Controller

STM32 NUCLEO-WB55RG with Mbed OS

LEVEL  Basic     |     TIME  60-90 minutes     |     PLATFORM  STM32 NUCLEO-WB55RG

Implement and test an automatic glass-door controller on the STM32 NUCLEO-WB55RG. The simulation provides a person detector and two end-position sensors. Your Mbed application commands opening and closing while protecting the mechanism at its endpoints.

The Door 3D simulation and original remote-laboratory exercise were created by WebLab-Deusto at the University of Deusto in Spain. This lesson adapts that activity to the STM32/Mbed workflow while preserving its operating problem, group-arrival behavior, reversal test, and sensor-fault extension. The completed application is provided separately to verified instructors.

Figure 1. Door 3D simulation by WebLab-Deusto, University of Deusto (Spain).

Figure 2. Feedback loop for the automatic door activity.

# Learning objectives

After completing the activity, you should be able to:

- translate physical control requirements into Boolean command rules;

- configure Mbed digital inputs and outputs for a remote simulation;

- use endpoint feedback to prevent an actuator from driving beyond a limit;

- implement break-before-make reversal between opposing commands;

- test normal operation, reversal, group arrival, and sensor faults;

- explain the difference between a requested motion and confirmed position.

# Prerequisites

You should know basic C++ control flow and Mbed DigitalIn and DigitalOut. You also need access to the NUCLEO-WB55RG Mbed Code-IDE and the REDTAIL Door simulation through your institution or laboratory provider.

Before starting, open the REDTAIL Door simulation and the STM32 mapping page.

# System behavior

The plant exposes three active-high inputs:

| Input | Meaning |
| --- | --- |
| doorOpened | The door has reached the fully open limit. |
| doorClosed | The door has reached the fully closed limit. |
| personSensor | At least one person is waiting or passing through the detection area. |

Your application produces two active-high outputs:

| Output | Meaning |
| --- | --- |
| open | Drive the door toward the open limit. |
| close | Drive the door toward the closed limit. |

The application must satisfy all of the following requirements:

- Open the door while a person is detected and the door is not fully open.

- Stop opening when the fully open sensor is active.

- Keep the door open while a person or group remains in the detection area.

- Close the door while the area is clear and the door is not fully closed.

- Stop closing when the fully closed sensor is active.

- If a person arrives while the door is closing, remove the close command before asserting the open command.

- Never assert open and close at the same time.

Unlike the VHDL version, this basic Mbed exercise uses a fast polling loop rather than requiring a state machine. The sensor feedback and output ordering still form a real closed-loop controller.

# STM32 signal mapping

All signals are active high. Directions are relative to your STM32 program.

| Runtime signal | Mbed pin | Direction | Purpose |
| --- | --- | --- | --- |
| Door fully open | PC_13 | Input | Stop the open command at the open endpoint. |
| Door fully closed | PA_6 | Input | Stop the close command at the closed endpoint. |
| Person detected | PB_9 | Input | Request opening and prevent closing. |
| Open command | PC_4 | Output | Drive the simulation toward fully open. |
| Close command | PD_0 | Output | Drive the simulation toward fully closed. |

Optional diagnostic LEDs are available on PB_13, PB_14, and PB_15. You may use them to mirror the person, opened, and closed inputs.

# Assignment

## 1. Derive the command rules

Write a two-row decision table for these modes:

- a person is detected;

- the detection area is clear.

For each mode, include the matching endpoint sensor in the command rule. Then verify from your table that the two commands cannot both be true.

## 2. Create the Mbed application

Create main.cpp in an Mbed OS 6 or Mbed OS CE project for NUCLEO_WB55RG. Your program should follow this structure without copying a completed command expression from another source:

#include "mbed.h"

using namespace std::chrono_literals;

int main() {

// Configure three DigitalIn objects and two DigitalOut objects.

// Initialize both actuator outputs low.

while (true) {

// Read one snapshot of the three inputs.

// Derive mutually exclusive open and close requests.

// Apply outputs using break-before-make ordering.

// Update optional diagnostic LEDs.

ThisThread::sleep_for(10ms);

}

}

Read each sensor once per loop into a local bool; do not repeatedly sample a changing input while deriving one output pair.

## 3. Apply outputs safely

When changing direction, deassert the previously active command before asserting the opposite command. A suitable control structure has three exclusive branches:

- opening requested;

- otherwise, closing requested;

- otherwise, stop.

Within the opening branch, clear close before setting open. Within the closing branch, clear open before setting close. This break-before-make ordering prevents even a brief software-generated overlap during reversal.

## 4. Compile and program

- Select the NUCLEO-WB55RG Mbed environment.

- Add or replace main.cpp in the project.

- Select the Door simulation.

- Compile the project and resolve every error or pin-name warning.

- Program the board and keep the simulation visible beside the board view.

# Required test sequence

Run the tests in order and record the observed sensor and command values.

| Test | Action | Expected behavior | Safety check |
| --- | --- | --- | --- |
| Initial state | Start with the door fully closed and no person. | The door remains stationary. | open = 0, close = 0 |
| Single arrival | Send one person while closed. | The door opens to the fully open limit. | Opening stops at doorOpened. |
| Clear doorway | Let the person leave after the door opens. | The door closes to the fully closed limit. | Closing stops at doorClosed. |
| Arrival while closing | Send another person before closing completes. | The door reverses safely and reopens. | close is cleared before open is set. |
| Group arrival | Keep the person sensor active for multiple arrivals. | The door stays fully open until the area is clear. | It does not close through the group. |
| Repeated cycles | Repeat the sequence several times. | Every cycle completes without getting stuck. | No output remains active at an endpoint. |

Use the optional LEDs to distinguish an input-mapping problem from an output logic problem. If the LEDs do not follow the simulation sensors, correct the input configuration before debugging the command rules.

# Sensor-fault extension

The original activity asks you to inject door-sensor errors and observe the result. If fault controls are available, test:

- both endpoint sensors high;

- neither endpoint sensor high while the door appears stationary;

- an endpoint signal that does not activate when the door reaches the limit.

For each case, record the commands produced by your program, decide whether the response is safe, and propose a conservative fault policy. Do not treat a contradictory sensor combination as a valid physical position.

# Deliverables

Submit:

- your command decision table;

- main.cpp with descriptive pin and command names;

- evidence for every required test, such as an observation table or instructor-approved demonstration;

- a brief explanation of endpoint interlocks and break-before-make reversal;

- your sensor-fault analysis.

# Optional challenges

- Add a non-blocking hold-open timer after the person sensor clears.

- Detect contradictory endpoint sensors and enter a conservative fault state.

- Record transitions over serial output without slowing the control loop.

- Refactor the policy into a pure function and unit-test every input combination on a development computer.

# Completion checklist

- The Mbed target is NUCLEO_WB55RG.

- All five pins match the REDTAIL mapping.

- Both actuator outputs initialize low.

- Each loop uses one consistent input snapshot.

- A group keeps the door open.

- A person arriving during closing causes a safe reversal.

- Both endpoint interlocks work.

- open and close are never high together, including during reversal.

- All required evidence and source files are ready to submit.